Empowering tomorrow’s leaders, today!

# Capstone Project 1: Secure Enterprise Network Setup with Virtualisation & Monitoring

Objective:

Design, configure, and secure a mid-sized enterprise network including servers, storage, and virtual environments with basic automation and monitoring.

Learning Outcomes

By completing this project, you will be able to:

- Design network architecture using VLANs and subnetting.

- Install and configure Windows/Linux servers and storage arrays.

- Apply basic security principles and auditing tools.

- Create and manage VMs and Docker containers.

- Implement basic automation and monitoring.

# Tasks

## 1. Network Design & Configuration

  - Design a 3-segment VLAN network: Admin, Sales, and IT.

- Cisco 2811 Router (1)

- Cisco 2960 switch (1)

- End Device - PC (9)

STEP 1: Add Network Device

STEP 2: Connections Made

  - Subnet each segment with CIDR.

- Assign IP Addresses and Default gateways

- Determine IP Address Class and Subnetting

VLAN 10 (Admin): 192.168.10.0/24

VLAN 20 (Sales): 192.168.20.0/24

VLAN 30 (IT): 192.168.30.0/24

- VLAN 10 - Admin: Default gateway: 192.168.10.1 Subnet Mask: 255.255.255.0

- VLAN 20 - Marketing: Default gateway: 192.168.20.1 Subnet Mask: 255.255.255.0

- VLAN 30 - IT: Default gateway: 192.168.10.1 Subnet Mask: 255.255.255.0

- Configure VLANs on switch

  - Implement basic routing between VLANs.

## 2. Server & Storage Setup

- Install a Linux (Ubuntu) and a Windows Server VM.

- Configure RAID 5 using virtual disks.

- To create a RAID 5 array open disk management. Right click the Disk 1 in the unallocated space and click New RAID 5 Volume option as shown below.

-  On the next screen that is displayed, select Disk 2 and click Add.

- The Next button will not be enabled as RAID 5 requires minimum 3 hard disks.

- Thus select Disk 3 and click Add. The final view should be as shown below. Then click Next

- Demonstrate file sharing between servers.

## 3. Security Implementation

- Set up a firewall (pfSense).

- Firewall rules

- Install and configure an IDS

- Downloading and Installing Snort

- Configuring IDS (Snort)

- Perform a basic vulnerability scan using Nmap or Nessus.

- Submit a 1-page audit report.

## 4. Virtualisation and Containers

- Create at least 2 Virtual Machines using VirtualBox or VMware.

- Install Docker and containerise a sample Python or Node.js web app.

## 5. Automation & Monitoring

- Use Ansible to automate file copy or service restart.

- Set up Nagios or Zabbix for monitoring CPU, RAM, and service health.

# Deliverables

- Network diagram

- Screenshots or screen recordings

- Docker container files and Ansible playbook

- Security audit report ● 	Final report with reflections